Table 1: Effect of Vbias1 without stacking keeping Vbias2 fixed at 0.9 V and Vdc at 1.6 V

| Vbias1 (V) | Power (µW) | Frequency (GHz) | Phase Noise (dBc/Hz) |
| --- | --- | --- | --- |
| 0.8 | 522.725 | 19.9927 | -62.5003 |
| 0.9 | 527.45 | 20.1041 | -63.1472 |
| 1 | 530.357 | 20.1732 | -63.2282 |
| 1.1 | 532.303 | 20.22 | -63.1244 |
| 1.2 | 533.671 | 20.255 | -63.1329 |
| 1.3 | 534.659 | 20.2802 | -63.131 |
| 1.4 | 535.385 | 20.2989 | -61.1292 |
| 1.5 | 535.922 | 20.3129 | -61.1278 |

Table 2: Effect of Vbias1 with stacking keeping Vbias2 fixed at 0.9 V and Vdc at 1.6 V

| Vbias1 (V) | Power (µW) | Frequency (GHz) | Phase Noise (dBc/Hz) |
| --- | --- | --- | --- |
| 0.9 | 407.674 | 19.98 | -56.6504 |
| 1 | 445.508 | 19.491 | -58.9723 |
| 1.1 | 463.499 | 19.459 | -59.6751 |
| 1.2 | 473.856 | 19.479 | -59.9704 |
| 1.3 | 480.581 | 19.5045 | -60.1352 |
| 1.4 | 485.228 | 19.526 | -60.2406 |
| 1.5 | 488.549 | 19.547 | -60.3097 |

Table 3: Effect of Vbias 2 without stacking keeping Vbias1 fixed at 0.9 V and Vdc at 1.6 V

| Vbias2 (V) | Power (µW) | Frequency (GHz) | Phase Noise (dBc/Hz) |
| --- | --- | --- | --- |
| 0.8 | 661.729 | 22.7939 | -62.2713 |
| 0.9 | 527.45 | 20.1041 | -63.1472 |
| 1 | 387.352 | 17.4574 | -63.051 |
| 1.1 | 253.759 | 14.2591 | -61.4196 |
| 1.2 | 137.215 | 10.1164 | -61.7157 |
| 1.3 | 50.8654 | 5.14366 | -60.2781 |

Table 4: Effect of Vbias2 with stacking keeping Vbias1 fixed at 0.9 V and Vdc at 1.6 V

| Vbias2 (V) | Power (µW) | Frequency (GHz) | Phase Noise (dBc/Hz) |
| --- | --- | --- | --- |
| 0.9 | 407.674 | 19.98 | -56.6504 |
| 1 | 338.339 | 15.8512 | -61.2348 |
| 1.1 | 232.055 | 13.1386 | -61.1897 |
| 1.2 | 129.136 | 9.49416 | -61.2515 |
| 1.3 | 49.1748 | 4.9263 | -60.0427 |
| 1.4 | 9.2248 | 1.3022 | -62.2013 |

Table 5: Effect of Vdc without stacking keeping Vbias1 and Vbias2 fixed at 0.9 V

| Vdc (V) | Power (µW) | Frequency (GHz) | Phase Noise (dBc/Hz) |
| --- | --- | --- | --- |
| 1.2 | 21.7079 | 3.25974 | -62.523 |
| 1.3 | 86.0012 | 8.41866 | -62.1545 |
| 1.4 | 196.985 | 13.2592 | -61.5721 |
| 1.5 | 346.625 | 17.066 | -61.2934 |
| 1.6 | 527.45 | 20.1041 | -63.1472 |

Table 6: Effect of Vdc with stacking keeping Vbias1 and Vbias2 fixed at 0.9 V

| Vdc (V) | Power (µW) | Frequency (GHz) | Phase Noise (dBc/Hz) |
| --- | --- | --- | --- |
| 1.2 | 21.2592 | 3.15371 | -63.0321 |
| 1.3 | 81.9662 | 7.96827 | -61.7326 |
| 1.4 | 181.942 | 12.3419 | -61.1577 |
| 1.5 | 304.282 | 15.711 | -60.9809 |
| 1.6 | 407.674 | 19.98 | -56.6504 |

%3CmxGraphModel%3E%3Croot%3E%3CmxCell%20id%3D%220%22%2F%3E%3CmxCell%20id%3D%221%22%20parent%3D%220%22%2F%3E%3CmxCell%20id%3D%222%22%20edge%3D%221%22%20parent%3D%221%22%20style%3D%22endArrow%3Dnone%3Bhtml%3D1%3Brounded%3D0%3B%22%20value%3D%22%22%3E%3CmxGeometry%20height%3D%2250%22%20relative%3D%221%22%20width%3D%2250%22%20as%3D%22geometry%22%3E%3CmxPoint%20x%3D%22170%22%20y%3D%22227.66%22%20as%3D%22sourcePoint%22%2F%3E%3CmxPoint%20x%3D%22230%22%20y%3D%22227.66%22%20as%3D%22targetPoint%22%2F%3E%3C%2FmxGeometry%3E%3C%2FmxCell%3E%3C%2Froot%3E%3C%2FmxGraphModel%3E

Fig1. Power Vs Vbias2 without Stacking

Fig2. Power Vs Vbias2 with Stacking

fFi

Fri

Fig 3. Power Vs Vbias1 without stacking

Fig 4. Power Vs Vbias1 with stacking

Fig 5. Power Vs Vdc without stacking

Fig 6. Power Vs Vdc with stacking